LoRa Water Quality Management System Development Log

6/20/2025 – Matthew Sharp:

- Identified preliminary BOM: Op-amps, digital pots, i2c accessories, LoRa chip.

- Narrowed LoRa communication scope to LoRa P2P (point to point) communication, removing overhead and extraneous features contained within the LoRaWAN protocol.

  - A custom network architecture likely preferred by Utility Companies as the data is less susceptible to third-party networking service issues.

  - Makes custom encryption and addressing of data much easier

- Located platform-agnostic Semtech sx1262 driver, requiring HAL (hardware abstraction layer) function definitions to be written by user

  - User must provide SPI calls within the HAL functions defined in the library

  - Makes initial development with the library much more straightforward – making platform switches away from pico much simpler.

    - Would ideally prefer to keep a HAL-based design for rest of project – demonstrating that with some minor firmware tweaks, the project is less susceptible to supply chain disturbances.

  - https://github.com/Lora-net/sx126x_driver/tree/v2.3.2

  - https://www.semtech.com/products/wireless-rf/lora-connect/sx1262

- Eliminated ATTiny85 as potential candidate for sensor node MCU

  - Though low power, inferior in program flash size, SPI accessibility, interrupt challenges, and potential for end-of-life.

  - Since it does not present itself as a COM port, complex systems are much more challenging to debug – no time or desire to debug SPI transactions and firmware issues with a $10 logic analyzer only.

- Preliminary choice of Raspberry Pi Pico 2 selected for MCU for the project, with eventual goal of doing an RP2350 layout on the final PCB.

  - More professional and modern solution. Higher processing capabilities and wider data bus are accommodating to overhead due to data encryption, watchdog timers, and interrupts.